CyanGem2 / Hey Cyan Smart Glasses

Technical Fix-Forward & Feature Enhancement Specification

HC-016 Build Scope: Gemini Live Bridge, Capture Workflow, Gallery/Media Bridge, and Glasses Control Guide

Core Product Truth Gemini Live is the primary AI brain. Hey Cyan glasses are the wearable Bluetooth microphone, speaker, and capture device. CyanGem2 is the support, diagnostics, workflow, launch, and media bridge app — not a replacement AI assistant.

| Prepared for | Juan Fuentes |
| --- | --- |
| Date | May 2, 2026 |
| Version | HC-016 Fix-Forward Specification |
| Recommended builder | Claude CoWork / Android Studio with Codex review after compile-clean build |

# Revision Control

| Version | Date | Owner | Notes |
| --- | --- | --- | --- |
| HC-016 | May 2, 2026 | Ops / Juan / Claude | Fix-forward scope for CyanGem2 after HC-015. Stabilizes four-tab app and aligns product around Gemini Live + Hey Cyan support workflow. |
| Future | TBD | Claude + Codex | BLE and Wi-Fi direct media sync spike only after core app compiles and launches cleanly. |

# Contents

1. Executive Summary

2. Current Product Direction

3. Hardware Control Reference

4. App and Architecture Decisions

5. HC-016 Screen Requirements

6. Android Technical Implementation

7. Gemini Live, Gemini API, and AICore Boundaries

8. Optional BLE / Wi-Fi Sync Spike

9. Claude Build Tasks

10. Acceptance Criteria

11. Juan Test Script

12. Risks and Guardrails

13. Roadmap

14. Appendix A: Claude Implementation Prompt

15. Appendix B: References and Source Notes

# 1. Executive Summary

This document defines a fix-forward technical plan for CyanGem2, Juan’s custom Android support app for Hey Cyan AI smart glasses. The goal is to stabilize the app, add truthful glasses workflow support, add a practical Gemini Live launch bridge, and add a Gallery/media bridge for photos and videos imported through the HeyCyan app.

Main Correction Treat the glasses as Bluetooth audio + physical controls + BLE-triggered capture + Wi-Fi/media import, not as a confirmed continuous live-stream camera. CyanGem2 should not advertise direct Gemini Live control or live glasses video until those paths are verified.

| What CyanGem2 Should Do | What CyanGem2 Should Not Do |
| --- | --- |
| Open Gemini and guide the user into Gemini Live. | Pretend to embed or control Gemini Live directly. |
| Show Bluetooth/glasses connection status truthfully. | Claim perfect connected state when Android can only infer it. |
| Teach and test the front button, rear button, and touchpad controls. | Send unknown BLE commands or factory reset-like commands. |
| Open HeyCyan for official media import and device management. | Replace HeyCyan before direct SDK sync is verified. |
| Show imported media from Android Gallery/MediaStore. | Promise automatic glasses media pull in HC-016. |

# 2. Current Product Direction

The app must stay focused. CyanGem2 is a support and workflow companion for the already-working Gemini Live + glasses setup. It should not drift back into custom chat, OpenRouter, Kimi, or a paid API-first assistant architecture.

| Component | Role |
| --- | --- |
| Gemini Live | Primary AI brain for live conversation and reasoning. |
| Hey Cyan smart glasses | Wearable Bluetooth microphone, speaker, and photo/video/audio capture hardware. |
| Samsung Galaxy phone | Connection hub, app host, Bluetooth audio route, and media bridge. |
| HeyCyan app | Official device management, firmware/app workflow, media import, and native AI features. |
| CyanGem2 | Support shell: launch Gemini/HeyCyan, show diagnostics, guide capture, bridge imported media, and document controls. |

Preserve exactly four runtime tabs: Home, Glasses, Gallery, and Settings.

Do not resurrect Chat, Ask Cyan, Gems, OpenRouter, Kimi, or the old custom AI answer engine.

Keep the app beginner-safe and truthful. When something is not directly controlled by CyanGem2, say so clearly.

Prioritize compile-clean, launch-clean, and Juan-testable behavior over advanced BLE/Wi-Fi automation.

# 3. Hardware Control Reference

Hey Cyan / G2PRO-style glasses place the primary controls on the right temple. The front button is commonly labeled A1. The rear AI button is commonly labeled A2. The flat exterior touchpad on the right temple handles media and volume gestures.

## 3.1 Front Button / A1

| Action | Expected Result |
| --- | --- |
| Long press 2-3 seconds | Power on. |
| Long press 5-6 seconds | Power off. |
| Single click | Capture photo. |
| Double click | Start video recording. |
| Single click during video | Stop and save video. |
| Single click on incoming call | Answer call. |
| Single click during call | End call. |
| Long press 2 seconds on incoming call | Reject call. |
| Long press 10 seconds | Factory reset / clear files. Do not expose this as a normal user action. |

## 3.2 Rear Button / A2 / AI Button

| Action | Expected Result |
| --- | --- |
| Single click | Wake the Hey Cyan native AI assistant. |
| Double click | Quick AI object recognition / snapshot recognition workflow. |
| Long press | Start audio/voice recording. |
| Short press while recording | Stop and save audio recording. |

Important Behavior Note The rear AI button appears to wake Hey Cyan’s native assistant or recognition workflow. It should not be described as a guaranteed Gemini wake button unless Gemini Live is already active and using glasses audio.

## 3.3 Right Temple Touchpad

| Gesture | Expected Result |
| --- | --- |
| Swipe forward toward lens | Volume up. |
| Swipe backward toward ear | Volume down. |
| Double tap | Play / pause music. |
| Long press about 2 seconds | Next track. |
| Triple tap | Previous track. |

## 3.4 Status Indicator Lights

| Indicator | Likely Meaning |
| --- | --- |
| Blinking blue | Searching, disconnected, or pairing behavior. |
| White flash | Photo capture confirmation. |
| Solid white | Video recording active. |
| Blinking white | Audio recording active. |
| Solid red | Charging. |
| Blue or off after charging | Charging complete, depending on model behavior. |

# 4. App and Architecture Decisions

## 4.1 Capture and Sync, Not Confirmed Live Camera

The safe architecture is capture-and-sync. The glasses can capture photos/videos/audio, then media is imported using the HeyCyan app or a future direct sync provider. Do not build a fake live camera screen or imply the glasses provide a continuous RTSP-style feed unless verified on Juan’s hardware.

| Layer | Purpose |
| --- | --- |
| Bluetooth Classic | Audio input/output: microphone, speaker, music, calls, and Gemini Live conversation audio. |
| BLE | Potential device commands/status: shutter, battery, media count, transfer mode, or button notifications. Future spike only. |
| Wi-Fi/AP transfer | High-bandwidth media import from glasses storage. Future direct implementation only after validation. |
| HeyCyan app | Official import and device management path. |
| CyanGem2 | Workflow, diagnostics, launch bridge, control guide, Gallery/media bridge. |

Do Not Build These in HC-016 FakeCameraLiveStreamService, RTSPGlassesPreview, ContinuousGlassesCameraView, OpenRouterChatScreen, AskCyanBrain, KimiFallback, or any paid API-first assistant layer.

Build These Instead GeminiLaunchBridge, GlassesStatusPanel, CaptureCheckScreen, HeyCyanImportGuide, MediaBridgeRepository, GalleryScreen, and an optional future BleControlSpike branch.

# 5. HC-016 Screen Requirements

## 5.1 Home Tab

The Home tab is the launch and workflow center. It should help Juan start the correct workflow quickly without pretending the app has powers it does not have.

| Section | Required Behavior |
| --- | --- |
| Connection header | Show Bluetooth enabled status, likely glasses connection, audio route hint, and Refresh button. |
| Gemini Live launch panel | Open Gemini, show Gemini Live instructions, and start the glasses workflow check. |
| HeyCyan app bridge | Open HeyCyan, Bluetooth settings, and app permissions/settings. |
| Capture Check | Guide user through photo, video, import, and return-to-gallery flow. |
| Truth card | Explain that live conversation uses Bluetooth audio and photos/videos require HeyCyan import unless direct sync is added later. |

| Button | Action |
| --- | --- |
| Open Gemini | Launch Gemini app or fallback to Play Store / friendly error. |
| Open HeyCyan | Launch HeyCyan app or fallback to Play Store / friendly error. |
| Open Bluetooth Settings | Launch Android Bluetooth settings. |
| Open Gallery | Navigate to CyanGem2 Gallery tab. |
| Run Capture Check | Open guided checklist. |
| Refresh Status | Re-run permission/Bluetooth/app-installed checks. |

## 5.2 Glasses Tab

The Glasses tab is the technical manual inside the app. It should be simple enough for live testing and detailed enough to support debugging.

Hardware Control Map: Front/A1, Rear/A2, touchpad gestures, and status lights.

Guided Test Mode: connect glasses, press front button once, confirm photo flash, double press to record video, single press to stop, then open HeyCyan import.

Troubleshooting Cards: Gemini hears phone instead of glasses, photos not appearing, HeyCyan import stuck, video issues, volume swipe not working, and rear button waking Hey Cyan instead of Gemini.

Limitations: Button remapping and direct camera control require BLE protocol validation and are not HC-016 requirements.

## 5.3 Gallery Tab

The Gallery tab bridges imported HeyCyan media into CyanGem2. It should use Android MediaStore and/or the Android Photo Picker rather than assuming a private HeyCyan storage path.

| Feature | Requirement |
| --- | --- |
| Recent photos | Show recent images from device Gallery / MediaStore. |
| Recent videos | Show recent videos from device Gallery / MediaStore. |
| Filter | Images, videos, or all media. |
| Manual picker | Let user choose a photo or video using Android Photo Picker. |
| Open/share | Open selected media in external viewer or share target where supported. |
| HeyCyan import path | Button to open HeyCyan and instructions to import/save media first. |
| Refresh | Re-query MediaStore. |
| Empty state | Explain that media must be imported from HeyCyan before CyanGem2 can see it. |

## 5.4 Settings Tab

| Setting / Section | Purpose |
| --- | --- |
| Preferred AI app | Gemini default. |
| HeyCyan package status | Installed / not installed. |
| Gemini package status | Installed / not installed. |
| Bluetooth permission status | Granted / missing. |
| Nearby devices permission | Granted / missing. |
| Media permission | Granted / missing. |
| Debug mode | Show package names, likely glasses device name, audio route hint, and recent media count. |
| About | Explain the CyanGem2 support-app role. |

Settings Guardrail Do not add OpenRouter, Kimi, API provider selection, chat model selection, or old AI brain settings.

# 6. Android Technical Implementation

## 6.1 Launcher Intents

object LauncherIntents { fun openGemini(context: Context) fun openHeyCyan(context: Context) fun openBluetoothSettings(context: Context) fun openAppSettings(context: Context) fun openGalleryPicker(context: Context) }

| Known / likely package | Use |
| --- | --- |
| com.google.android.apps.bard | Gemini app candidate. |
| com.google.android.googlequicksearchbox | Google app fallback candidate. |
| com.glasssutdio.wear | HeyCyan app package from Google Play listing. |

1. Try explicit package launch.

2. Fall back to generic app intent if needed.

3. Fall back to Play Store search / app details.

4. Show a friendly message if the app is missing.

## 6.2 Bluetooth Status Checker

sealed class GlassesConnectionStatus { object Unknown : GlassesConnectionStatus() object BluetoothOff : GlassesConnectionStatus() object PermissionMissing : GlassesConnectionStatus() object NotFound : GlassesConnectionStatus() data class Found(val deviceName: String, val addressMasked: String) : GlassesConnectionStatus() data class ConnectedLikely(val deviceName: String) : GlassesConnectionStatus() }

| Check | Implementation Note |
| --- | --- |
| Bluetooth adapter enabled | BluetoothAdapter / BluetoothManager. |
| Bonded/connected devices | Use available profile proxies where possible. |
| Device name match | Search for Hey, Cyan, W630, CK, G2, G3, BE610, M02, Padmate. |
| Audio route hint | Use AudioManager as a hint, not absolute truth. |
| Permission status | Show missing Bluetooth/Nearby Devices permissions clearly. |

Bluetooth Truth Rule If Android cannot confirm an active audio profile connection, say “likely connected” or “found,” not “guaranteed connected.”

## 6.3 Android Permissions

<uses-permission android:name="android.permission.BLUETOOTH_CONNECT" /> <uses-permission android:name="android.permission.BLUETOOTH_SCAN" /> <uses-permission android:name="android.permission.ACCESS_FINE_LOCATION" android:maxSdkVersion="30" /> <uses-permission android:name="android.permission.READ_MEDIA_IMAGES" /> <uses-permission android:name="android.permission.READ_MEDIA_VIDEO" /> <uses-permission android:name="android.permission.READ_EXTERNAL_STORAGE" android:maxSdkVersion="32" />

Prefer Android Photo Picker where possible, especially on newer Android versions, to avoid permission drama. MediaStore permissions should be requested gracefully when the app needs a recent-media grid.

## 6.4 Media Bridge Repository

data class CyanMediaItem( val uri: Uri, val displayName: String?, val mimeType: String, val dateAdded: Long?, val sizeBytes: Long?, val sourceHint: MediaSourceHint ) enum class MediaSourceHint { DEVICE_GALLERY, USER_PICKED, HEYCYAN_IMPORTED_UNKNOWN, UNKNOWN }

| Function | Description |
| --- | --- |
| getRecentImages() | Query MediaStore images. |
| getRecentVideos() | Query MediaStore videos. |
| getRecentMedia() | Combined image/video list. |
| refresh() | Re-query MediaStore. |
| openPicker() | Launch Android Photo Picker. |
| shareToGemini() | Share selected media via Android ACTION_SEND if supported. |
| openInExternalViewer() | Let Android choose viewer. |

# 7. Gemini Live, Gemini API, and AICore Boundaries

## 7.1 Safe Gemini Live Bridge Now

| Capability | HC-016 Status | Notes |
| --- | --- | --- |
| Open Gemini app | Yes | Use launcher intent. |
| Guide user into Gemini Live | Yes | Instruction card only. |
| Route audio through glasses | Mostly user/device controlled | Relies on Bluetooth audio already working. |
| Share imported media to Gemini | Maybe | Depends on Gemini app share target behavior. |
| Embed Gemini Live UI | No | Not supported as normal app embed. |
| Control Gemini Live session | No | Requires Gemini Live API architecture, not app UI control. |

Recommended Interpretation of “Gemini Live In-Frame” For HC-016, “in-frame” should mean CyanGem2 guides and launches the Gemini Live workflow while Gemini remains in Google’s app or overlay. A custom Live API client is a later advanced phase.

## 7.2 Future Gemini Live API Path

The Gemini Live API is a stateful WebSocket-based path for low-latency audio and visual input, including raw PCM audio and image/video-style frames. This is powerful but should be treated as a later phase because it requires API/security/cost decisions, token handling, audio routing, and stronger privacy controls.

Use only after the app shell, Gallery bridge, and capture workflow are stable.

Consider LiveKit or Pipecat wrappers later to avoid writing all audio jitter-buffer and interruption logic from scratch.

Use ephemeral tokens or a backend broker if moving toward production.

## 7.3 On-Device Gemini Nano / AICore

Gemini Nano through Android AICore / ML Kit GenAI can be useful for future local image understanding, but it should not be treated as a drop-in replacement for Gemini Live.

| Use Case | Fit |
| --- | --- |
| Quick image description from imported photo | Good future fit. |
| Basic classification or text extraction | Good future fit. |
| Offline-ish helper | Possible, device/API dependent. |
| Full Gemini Live conversation replacement | Not a fit. |
| Continuous glasses live vision | Not a fit unless camera stream exists. |

# 8. Optional BLE / Wi-Fi Sync Spike

The community SDK and Wi-Fi transfer research may become useful later, but HC-016 should not depend on it. This should be isolated in a future feature branch after compile-clean app stability.

## 8.1 BLE Control Spike

| Spike Goal | Guardrail |
| --- | --- |
| Scan for glasses | Do not require this for normal HC-016 app use. |
| Identify advertised device names | Use flexible name matching, not one hardcoded value. |
| Read battery if standard service exists | Safe read only. |
| Test SDK sample | Only if legal/buildable and isolated. |
| Test photo command | Only after Juan confirms and only known safe commands. |
| Avoid factory reset / unknown hex | Absolutely no blind command writing. |

Future branch: feature/ble-control-spike Potential community-reported UUIDs: Primary Service: 7905FFF0-B5CE-4E99-A40F-4B1E122D00D0 Secondary Service: 6e40fff0-b5a3-f393-e0a9-e50e24dcca9e Treat these as discovery hints, not production guarantees.

## 8.2 Wi-Fi Media Sync Spike

Future direct media sync should use a provider interface so the current manual HeyCyan import path remains stable.

interface GlassesMediaSyncProvider { suspend fun isSupported(): Boolean suspend fun openTransferMode(): TransferModeResult suspend fun listMedia(): List<RemoteGlassesMedia> suspend fun download(media: RemoteGlassesMedia): Uri } class HeyCyanManualImportProvider : GlassesMediaSyncProvider class BleWifiDirectSyncProvider : GlassesMediaSyncProvider // future only

HC-016 Decision Implement HeyCyanManualImportProvider behavior only. Direct BLE/Wi-Fi sync is a future spike after the base app works.

# 9. Claude Build Tasks

| Task Group | Claude Should Implement |
| --- | --- |
| A — Stabilize App Shell | Four tabs only, no old AI/chat runtime nav, compile-clean, launch-clean, dark/cyan visual design. |
| B — Home Screen | Gemini launch card, HeyCyan launch card, Bluetooth/glasses status, Capture Check, limitations card. |
| C — Glasses Screen | Hardware control map, touchpad actions, status indicators, guided button test, troubleshooting. |
| D — Gallery Screen | MediaStore and/or Photo Picker, recent media display, empty state, refresh, open/share selected media, open HeyCyan. |
| E — Settings Screen | App installed checks, permissions, debug/about info, no API/provider settings. |
| F — Helpers | LauncherIntents.kt, PackageDetector.kt, BluetoothStatusChecker.kt, MediaBridgeRepository.kt, CyanMediaItem.kt. |

# 10. Acceptance Criteria

| Check | Expected Result |
| --- | --- |
| Android Studio Make Project | Pass. |
| App install | Pass. |
| App launch | No crash. |
| Bottom nav | Home, Glasses, Gallery, Settings only. |
| Old AI/chat tabs | Not visible. |
| Theme | Dark/cyan and readable. |
| Home buttons | Do not crash; use graceful fallbacks. |
| Bluetooth status | Shows truthful state. |
| HeyCyan launch | Opens app or fallback message. |
| Gemini launch | Opens Gemini or fallback message. |
| Gallery | Shows recent media or clear empty state. |
| Permissions | Requests gracefully. |
| No fake claims | App does not say it directly controls Gemini Live or confirmed live glasses camera. |

# 11. Juan Test Script

## Test 1 — Launch

Open CyanGem2. Confirm it does not crash. Confirm bottom tabs are Home, Glasses, Gallery, Settings. PASS if app opens and tabs are correct. FAIL if crash or old Chat/OpenRouter/Kimi screens appear.

## Test 2 — Gemini Bridge

Tap Open Gemini. Start Gemini Live manually. Talk through Hey Cyan glasses. Walk around like before. PASS if Gemini hears/responds through glasses. FAIL if audio switches to phone unexpectedly.

## Test 3 — Glasses Controls

Go to Glasses tab. Press front button once. Confirm photo indicator flash. Double press front button. Confirm video starts. Single press front button. Confirm video stops. PASS if physical controls behave as documented. FAIL if button behavior differs; update the control map.

## Test 4 — HeyCyan Import

Tap Open HeyCyan. Go to HeyCyan album/import. Import latest photo/video. Save to phone album if needed. PASS if media imports and appears in phone gallery. FAIL if import fails or media is stuck inside HeyCyan only.

## Test 5 — CyanGem2 Gallery

Return to CyanGem2. Open Gallery tab. Tap Refresh. Look for imported media. PASS if imported media appears or picker can select it. FAIL if app cannot access media even after permission granted.

# 12. Risks and Guardrails

| Risk | Guardrail |
| --- | --- |
| Fake Gemini integration | Say CyanGem2 launches/guides Gemini. Do not say it controls Gemini Live directly. |
| Fake live camera | Say capture/import/Gallery bridge. Do not claim live glasses camera feed. |
| Unofficial SDK instability | Use SDK only in a future spike branch after app stability. |
| Android permissions | Prefer Photo Picker where possible; request MediaStore permissions gracefully. |
| Voice shortcuts overpromising | Treat App Actions/shortcuts.xml as future enhancement, not HC-016 requirement. |
| BLE command danger | No unknown hex commands. No factory reset paths. No blind writes. |

# 13. Roadmap

| Phase | Feature Set |
| --- | --- |
| Phase 1 — Stabilized Support App | Four tabs, Gemini launch, HeyCyan launch, Bluetooth status, glasses manual/control map, Gallery bridge, capture checklist. |
| Phase 2 — Smarter Media Bridge | Better HeyCyan media detection, watch folder, auto-refresh after returning from HeyCyan, share-to-Gemini improvements. |
| Phase 3 — BLE Control Spike | BLE scan, battery read, media count read, remote photo capture, video/audio control, Wi-Fi transfer mode. |
| Phase 4 — AI Vision Layer | Gemini Nano image prompt, Gemini API image analysis, Gemini Live API, LiveKit/Pipecat bridge, ephemeral-token backend. |

# Appendix A: Claude Implementation Prompt

Paste the following prompt to Claude CoWork for HC-016 implementation.

You are working on CyanGem2, Juan’s Hey Cyan smart glasses support app. This is HC-016 fix-forward from the post-HC-015 state. Do not roll back. Do not create a new repo. Do not resurrect Chat, Ask Cyan, Gems, OpenRouter, Kimi, or any custom AI answer engine. Product truth: Gemini Live is the primary AI brain. Hey Cyan glasses are the Bluetooth mic/speaker and capture hardware. CyanGem2 is a support/control/workflow/diagnostics/media bridge app. Preserve four tabs only: Home Glasses Gallery Settings Implement or repair: 1. HomeScreen: - Gemini launch card - HeyCyan launch card - Bluetooth/glasses status card - Capture Check card - truthful limitations card 2. GlassesScreen: - hardware control map for Front/A1, Rear/A2, touchpad - status indicators - guided button test - troubleshooting cards 3. GalleryScreen: - query recent images/videos using MediaStore and/or Android Photo Picker - show empty state explaining HeyCyan import - button to open HeyCyan - refresh button - open/share selected media if possible 4. SettingsScreen: - app installed checks for Gemini and HeyCyan - permission status - debug/about info - no API key/model/provider settings 5. Helper files: - LauncherIntents.kt - PackageDetector.kt - BluetoothStatusChecker.kt - MediaBridgeRepository.kt - CyanMediaItem.kt Important constraints: - Do not claim CyanGem2 directly controls Gemini Live. - Do not claim live camera streaming from glasses. - Treat photos/videos as capture + HeyCyan import + Gallery/MediaStore bridge. - BLE/Wi-Fi direct control is future spike only unless existing project already has safe code. - Make project compile clean. - Keep visual design dark/cyan, not the accidental HC-015 light theme. - Provide a final summary listing changed files, what was implemented, and exact Android Studio tests Juan should run.

# Appendix B: References and Source Notes

These references were used to ground the specification. Some device-level details come from G2PRO/HeyCyan-compatible manuals and community SDK research, so exact behavior should still be validated against Juan’s physical glasses.

G2PRO Smart Glasses User Manual: Documents A1/A2 controls, touchpad actions, photo/video/audio behavior, HeyCyan app import flow, indicators, and AI wake/object recognition. https://device.report/manual/18778804

HeyCyan app listing on Google Play: Confirms HeyCyan app role for device management, gallery import/view/share, and supported smart glasses workflow. https://play.google.com/store/apps/details?id=com.glasssutdio.wear&hl=en_US

HeyCyan Smart Device FAQ: Official FAQ page exists, but the smart device FAQ content was sparse/empty when checked. https://heycyan.net/docs/smart-device-faq

Community HeyCyan Smart Glasses SDK: Unofficial/community SDK with BLE/Wi-Fi architecture hints. Treat as research, not production guarantee. https://github.com/ebowwa/HeyCyanSmartGlassesSDK

Google Gemini Live API documentation: Describes stateful WebSocket Live API and real-time audio/visual interaction path for future advanced phase. https://ai.google.dev/api/live

Gemini API Live API overview: Describes low-latency real-time voice/video capabilities, modality support, and partner options such as LiveKit/Pipecat. https://ai.google.dev/gemini-api/docs/live-api

Google ML Kit GenAI documentation: Documents on-device Gemini Nano / AICore style generative AI APIs for future local image/text analysis paths. https://developers.google.com/ml-kit/genai

Android Wi-Fi Network Suggestion Builder: Official Android API reference for future Wi-Fi media sync spike. https://developer.android.com/reference/android/net/wifi/WifiNetworkSuggestion.Builder

Android App Actions getting started: Reference for future shortcuts/App Actions voice-trigger path. https://developer.android.com/develop/devices/assistant/get-started